# System Design & Architecture

This document explains the end-to-end architecture of the Eastman Inverter Monitoring System.

## High-Level Architecture Flow

The system consists of four main layers: The Source (Inverter), The Bridge (Hardware), The Server (ESP32), and The Client (Flutter App).

graph TD
    A[Eastman Inverter] -- RS485 Protocol --> B(RS485 to TTL Module)
    B -- UART / TTL Serial --> C{ESP32 Microcontroller}
    C -- Local WiFi Network --> D[Flutter Mobile App]
    
    subgraph Hardware Setup
    A
    B
    C
    end
    
    subgraph Software Layer
    D
    end

## Physical Wiring & Connections

### 1. Inverter to RS485 Module

The inverter exposes an RS485 port (often a 4-pin circular connector or RJ45). RS485 uses differential signaling over two wires, usually labeled A and B. * Inverter Pin 1 (A/D+) connected to RS485 Module Pin A * Inverter Pin 2 (B/D-) connected to RS485 Module Pin B

### 2. RS485 Module to ESP32

The RS485 module translates the robust RS485 voltages into TTL logic levels (0-3.3V) that the ESP32 can safely read. * VCC: Connected to ESP32 VIN or 5V (powered by the mobile charger). * GND: Connected to ESP32 GND. * RXD (Receiver): Connected to ESP32 Pin 27 (Software configured as RX for Serial2). * TXD (Transmitter): Connected to ESP32 Pin 26. (Note: In our final passive-listener design, the TXD line is kept high and unused since we don’t send data back).

### 3. Power Supply

- The ESP32 is powered via its micro-USB port using a standard OnePlus 5V Mobile Wall Charger.

- The ESP32’s onboard voltage regulator steps this 5V down to 3.3V for its own chip, while passing the 5V directly to the RS485 module via the VIN pin.

## Software Data Flow

sequenceDiagram
    participant Inv as Eastman Inverter
    participant ESP as ESP32 (Hardware Serial2)
    participant Mem as ESP32 (Internal Memory)
    participant App as Flutter App

    loop Every second
        Inv->>ESP: Broadcasts 26-Byte Hex Frame (115200 baud)
    end
    
    loop Continuous ESP32 Loop
        ESP->>ESP: Read Serial Buffer
        ESP->>ESP: Detect 0xFFFF Header
        ESP->>Mem: Parse 26 bytes & Convert Hex to Floats
    end

    loop Every 3 seconds
        App->>Mem: GET http://192.168.31.127/telemetry
        Mem-->>App: Respond with JSON { "batteryVoltage": 51.6, ... }
    end

## How the Parsing Works

- Header Detection: The ESP32 reads bytes one by one until it sees 0xFF followed immediately by another 0xFF. This marks the start of a new data package.

- Buffer Collection: It then captures the next 24 bytes, filling a 26-byte array.

- Bit-shifting: The data is sent in two-byte pairs (16-bit integers). The ESP32 combines them using bitwise operations. For example, if the Battery Voltage bytes are 0x13 and 0x33:

  - Combine: 0x1333 (which is 4915 in decimal).

  - Scale: The manual states voltage is scaled by 100, so 4915 / 100 = 49.15 Volts.

- JSON Serving: These decoded values are stored in a struct. When the Flutter app makes an HTTP GET request, the ESP32 formats this struct into a JSON string and sends it over the local network.